# CUDA + C++ Development Environment Setup — Windows 11

End-to-end setup notes for this project on Windows 11, mirroring SETUP.md (Linux) but Windows-only. Every step here is what you actually need to type or click on a fresh Windows machine to go from nothing to a working CUDA build.

The project itself (CMakeLists.txt, src/, include/, .clang-format, .gitignore) is identical across platforms — only the toolchain around it differs.

## 1. Hardware discovery — what GPU do you have?

On a brand-new laptop, do this first. Don’t install anything until you’ve confirmed: 1. There is an NVIDIA GPU at all (some “gaming-looking” laptops are integrated-graphics-only). 2. The exact model and architecture. 3. That architecture is still supported by current NVIDIA drivers. 4. How hybrid graphics is set up (most NVIDIA laptops have two GPUs).

These steps don’t require any NVIDIA software — Windows can identify the hardware on its own.

### 1.1 Identify the GPU (five methods, pick whichever)

Method A — Task Manager (fastest, no clicks beyond opening it).

Ctrl + Shift + Esc → Performance tab → look in the left sidebar for GPU 0, GPU 1, etc. Each entry shows the model name in the top-right (“NVIDIA GeForce RTX 4060 Laptop GPU”). If you see two GPUs (e.g. “Intel Iris Xe” and an NVIDIA one), you have hybrid graphics — see §1.4.

Method B — Device Manager.

Win + X → Device Manager → expand Display adapters. Each adapter is listed by exact name. Right-click → Properties → Details tab → Property: Hardware Ids gives you the PCI vendor/device ID (PCI\VEN_10DE&DEV_xxxx — 10DE is NVIDIA).

Method C — DirectX Diagnostic Tool (dxdiag).

Win + R → type dxdiag → Enter → Display tab(s). Shows manufacturer, chip type, driver version, and DirectX feature level. Multiple Display tabs = multiple GPUs.

Method D — System Information (msinfo32).

Win + R → msinfo32 → Components → Display. Same data, exportable to a text file via File → Save.

Method E — PowerShell (scriptable, works in a script or remote session).

Get-CimInstance Win32_VideoController |
  Select-Object Name, AdapterRAM, DriverVersion, VideoProcessor |
  Format-Table -AutoSize

Or, by PCI ID (works even when no driver is bound):

Get-PnpDevice -Class Display |
  Select-Object FriendlyName, InstanceId, Status

The InstanceId contains the VEN_10DE&DEV_xxxx segment — the four hex digits after DEV_ are the device ID. Cross-reference at https://pci-ids.ucw.cz/read/PC/10de if Windows shows only “Microsoft Basic Display Adapter” (i.e. no driver yet).

### 1.2 Look up the compute capability

Once you have the model name, look it up at NVIDIA’s official list: https://developer.nvidia.com/cuda-gpus.

You’ll get a compute capability number (like 8.9). The integer form (89) is what you pass to CMake as -DCMAKE_CUDA_ARCHITECTURES=89.

Quick reference for common laptop GPUs you might see in 2026:

| Marketing name | Architecture | Compute capability | Driver-support status |
| --- | --- | --- | --- |
| GTX 9-series Mobile (940M, 950M, 960M…) | Maxwell | 5.0 / 5.2 | Legacy — supported only by the 470 LTS driver. CUDA 12 dropped Maxwell. |
| GTX 10-series Mobile (1050, 1050 Ti, 1060, 1070, 1080) | Pascal | 6.1 | Supported through driver branch 580 (LTS); dropped in 590+. CUDA 13 dropped Pascal — stay on CUDA 12.x. |
| MX 150 / 250 / 350 | Pascal | 6.1 | Same as above. |
| GTX 16-series (1650, 1660 Ti Mobile) | Turing | 7.5 | Fully supported, current. |
| RTX 20-series Mobile (2060, 2070, 2080) | Turing | 7.5 | Fully supported, current. |
| MX 450 / 550 | Turing | 7.5 | Fully supported. |
| RTX 30-series Mobile (3050, 3060, 3070, 3080 Ti…) | Ampere | 8.6 | Fully supported, current. |
| RTX 40-series Mobile (4050, 4060, 4070, 4080, 4090) | Ada Lovelace | 8.9 | Fully supported, current. |
| RTX 50-series Mobile (5060, 5070, 5080, 5090) | Blackwell | 12.0 | Fully supported, requires recent driver (≥570) and CUDA ≥12.8. |

If your card isn’t in this short list, the official page above is authoritative.

### 1.3 Is your GPU still supported?

Two questions to answer:

A. Is the architecture supported by the current driver branches?

| Architecture | Current driver support (2026) | What this means for you |
| --- | --- | --- |
| Kepler (older 6/7-series) | None — last in driver 470 (legacy, 2024 EOL) | CUDA dev not practical; consider CPU-only or upgrade hardware. |
| Maxwell | Driver 470 LTS only | OK for legacy CUDA 11.x. Skip CUDA 12+. |
| Pascal (10-series) | Driver branches 535–580 (580 is LTS through ~2028); dropped in 590+ | Use CUDA 12.x and pin to driver ≤580. |
| Turing, Ampere, Ada, Hopper, Blackwell | All current branches | No restrictions; use the latest CUDA. |

B. Is it CUDA-capable at all?

Almost every GeForce, Quadro, RTX, and Tesla card from the last decade is. The exceptions are GeForce MX 110/130 rebranded older Maxwell parts and very old GT-series chips. The list at https://developer.nvidia.com/cuda-gpus is definitive — if your card is listed, CUDA works on it.

### 1.4 Hybrid graphics (NVIDIA Optimus) — laptops only

This trips up almost everyone the first time. Most NVIDIA laptops have two GPUs: - An integrated GPU (Intel Iris Xe or AMD Radeon Graphics) that drives the desktop and runs background apps to save battery. - A discrete NVIDIA GPU that activates only for graphics-heavy or compute apps.

Implications for CUDA:

- A CUDA program will use the discrete NVIDIA GPU automatically — that’s where the CUDA driver is. You don’t have to do anything special at runtime.

- But Windows may keep the discrete GPU powered down until something asks for it. nvidia-smi waking it up takes a second or two; that’s normal.

- Some IDEs / editors get pinned to the integrated GPU by Windows’ graphics preferences. If you want VS Code or Nsight to use the NVIDIA GPU (e.g. for hardware-accelerated rendering), set it manually: Settings → System → Display → Graphics → Browse → pick the app → Options → High performance.

To confirm both GPUs are visible without installing the driver:

Get-CimInstance Win32_VideoController | Select-Object Name, AdapterRAM

You should see two rows: one Intel/AMD, one NVIDIA. If you see only the integrated one, the NVIDIA part is either disabled in BIOS, broken, or this is a non-hybrid laptop.

To check BIOS if the NVIDIA GPU is missing entirely: reboot, enter BIOS/UEFI setup (usually F2, Del, or Esc at boot), look for Graphics, Display, or MUX Switch settings. Some gaming laptops have a “discrete GPU only” mode that disables the integrated GPU and routes the display through the NVIDIA card directly — better for CUDA work, worse for battery.

### 1.5 Basic environment checklist

| Question | Answer |
| --- | --- |
| Disk space? | ~10 GB total — VS Build Tools ~6 GB; CUDA toolkit ~3 GB. |
| Admin account? | Yes — installer needs to write to Program Files and load drivers. |
| Internet? | Yes — winget pulls everything from official sources. |
| Windows up to date? | Recommended — Settings → Windows Update before installing the NVIDIA driver. |
| winget works? | Test in PowerShell: winget --version. If “command not found,” install App Installer from the Microsoft Store. |

## 2. Recommended stack

| Layer | Choice | Rationale |
| --- | --- | --- |
| Host compiler | MSVC (Visual Studio 2022 Build Tools) | The only host compiler nvcc officially supports on Windows. MinGW and Clang are not supported. |
| CUDA Toolkit | Latest 12.x that matches your GPU’s arch and your driver | If you have a Pascal (10-series) card, stick to 12.x — CUDA 13.x dropped Pascal. Modern cards can use the latest. |
| NVIDIA driver | Latest Game Ready or Studio via the NVIDIA App | Higher than the toolkit’s minimum. Windows handles forward compatibility well. |
| C++ standard | C++20 | nvcc 12.6 supports it; modern default. |
| Build system | CMake ≥ 3.24 + Ninja | First-class CUDA language support; faster incremental builds than VS multi-config. |
| IDE | VS Code + “Nsight Visual Studio Code Edition” | Free, official NVIDIA extension with CUDA-GDB integration. The full Visual Studio IDE also works (with Nsight Visual Studio Edition) if you prefer it. |
| Profilers | Nsight Systems + Nsight Compute | Bundled with the toolkit. Both have full Windows GUIs. |
| Style / lint | clang-format | Ships with VS Build Tools’ LLVM components, or via winget LLVM.LLVM. |

## 3. Manual install (reference)

The automated script in §4 does all of this. This section is the “what’s actually happening” reference.

### 3.1 NVIDIA driver

Easiest: install the NVIDIA App, which manages driver updates with one click.

winget install --id Nvidia.NVIDIAApp

Open it, sign in (optional), pick Drivers tab, install the latest Game Ready (gaming/general) or Studio (creative/CUDA workloads) driver. Reboot afterward.

Alternative: download directly from https://www.nvidia.com/Download/index.aspx.

### 3.2 Visual Studio 2022 Build Tools (MSVC + Windows SDK)

You need MSVC’s cl.exe. You don’t need the full Visual Studio IDE — Build Tools is the lightweight option (~6 GB vs ~10 GB).

winget install --id Microsoft.VisualStudio.2022.BuildTools --override `
  "--passive --wait --add Microsoft.VisualStudio.Workload.VCTools `
   --add Microsoft.VisualStudio.Component.Windows11SDK.22621 `
   --includeRecommended"

If you’d rather use the GUI: download from https://visualstudio.microsoft.com/downloads/ → Build Tools for Visual Studio 2022 → in the installer, check Desktop development with C++. Make sure the right pane includes the MSVC v143 toolset and a Windows 11 SDK.

### 3.3 CUDA Toolkit

winget install --id Nvidia.CUDA

Or grab a specific version’s .exe from https://developer.nvidia.com/cuda-downloads (Windows → x86_64 → 11 → exe, local or network installer).

After install, verify in a new shell:

nvcc --version

The installer adds C:\Program Files\NVIDIA GPU Computing Toolkit\CUDA\v12.x\bin to the system PATH automatically.

### 3.4 Build tools

winget install --id Kitware.CMake
winget install --id Ninja-build.Ninja
winget install --id Git.Git

### 3.5 VS Code + CUDA extensions

winget install --id Microsoft.VisualStudioCode
code --install-extension nvidia.nsight-vscode-edition
code --install-extension ms-vscode.cpptools
code --install-extension ms-vscode.cmake-tools

The code CLI may not be on PATH until you open a new shell after installing VS Code.

## 4. Automated install script

Saved as scripts/install.ps1. Runs all of §3 idempotently. Driver install is opt-in (-InstallDriver) since you’ll usually pick the driver yourself in the NVIDIA App.

### Usage

Open an elevated PowerShell (Right-click PowerShell → Run as Administrator), then:

# Allow this session to run a local script (the policy reverts when you close the shell)
Set-ExecutionPolicy -Scope Process -ExecutionPolicy Bypass

# All defaults (no driver, includes VS Code)
.\scripts\install.ps1

# Also install the NVIDIA App for driver management
.\scripts\install.ps1 -InstallDriver

# Skip VS Code if you use a different IDE
.\scripts\install.ps1 -SkipVSCode

# Help
Get-Help .\scripts\install.ps1 -Detailed

### Script contents

<#
.SYNOPSIS
    Installer for the cuda_playground toolchain on Windows 11.

.DESCRIPTION
    Installs:
      - Visual Studio 2022 Build Tools (MSVC + Windows 11 SDK)  -- required by nvcc on Windows
      - CUDA Toolkit                                             -- via winget
      - CMake, Ninja, Git                                        -- via winget
      - VS Code + CUDA extensions (Nsight, C/C++, CMake Tools)   -- via winget

    The NVIDIA driver itself is opt-in (-InstallDriver). On Windows, drivers
    are usually managed by the NVIDIA App or Windows Update; the script only
    installs the NVIDIA App and tells you to open it.

.PARAMETER InstallDriver
    Install the NVIDIA App so you can pick a Game Ready / Studio driver.

.PARAMETER SkipVSCode
    Skip VS Code if you use a different IDE.

.EXAMPLE
    .\scripts\install.ps1

.EXAMPLE
    .\scripts\install.ps1 -InstallDriver

.NOTES
    Run from an *elevated* PowerShell ("Run as Administrator").
    After install, build from the "x64 Native Tools Command Prompt for VS 2022".
#>

[CmdletBinding()]
param(
    [switch]$InstallDriver,
    [switch]$SkipVSCode
)

$ErrorActionPreference = 'Stop'
Set-StrictMode -Version Latest

function Write-Log  { param([string]$m) Write-Host "[install] $m" -ForegroundColor Cyan }
function Write-Warn { param([string]$m) Write-Host "[warn]    $m" -ForegroundColor Yellow }
function Die        { param([string]$m) Write-Host "[error]   $m" -ForegroundColor Red; exit 1 }

# ---------------------------------------------------------------------------
# Pre-flight
# ---------------------------------------------------------------------------
$principal = [Security.Principal.WindowsPrincipal]::new(
    [Security.Principal.WindowsIdentity]::GetCurrent())
if (-not $principal.IsInRole([Security.Principal.WindowsBuiltInRole]::Administrator)) {
    Die "Run from an elevated PowerShell prompt (Right-click PowerShell -> Run as Administrator)."
}

if (-not (Get-Command winget -ErrorAction SilentlyContinue)) {
    Die "winget not found. Install 'App Installer' from the Microsoft Store, then re-run."
}

$wingetCommon = @('--accept-source-agreements', '--accept-package-agreements', '--silent')

function Install-Winget {
    param([string]$Id, [string]$Display = $Id)
    Write-Log "installing $Display ($Id)"
    & winget install --id $Id @wingetCommon
    if ($LASTEXITCODE -ne 0 -and $LASTEXITCODE -ne -1978335189) {
        # -1978335189 = APPINSTALLER_CLI_ERROR_NO_APPLICABLE_UPDATE_FOUND (already installed)
        Write-Warn "winget install for $Id returned exit $LASTEXITCODE (continuing)"
    }
}

# ---------------------------------------------------------------------------
# 1. Visual Studio Build Tools 2022 (MSVC + Windows 11 SDK)
# ---------------------------------------------------------------------------
Write-Log "installing Visual Studio 2022 Build Tools (MSVC + Windows 11 SDK)"
& winget install --id Microsoft.VisualStudio.2022.BuildTools `
    --override "--passive --wait --add Microsoft.VisualStudio.Workload.VCTools --add Microsoft.VisualStudio.Component.Windows11SDK.22621 --includeRecommended" `
    @wingetCommon
if ($LASTEXITCODE -ne 0 -and $LASTEXITCODE -ne -1978335189) {
    Write-Warn "VS Build Tools install returned exit $LASTEXITCODE"
}

# ---------------------------------------------------------------------------
# 2. NVIDIA driver (opt-in)
# ---------------------------------------------------------------------------
if ($InstallDriver) {
    Install-Winget 'Nvidia.NVIDIAApp' 'NVIDIA App (driver manager)'
    Write-Warn "Open the NVIDIA App and install the latest Game Ready or Studio driver."
    Write-Warn "A reboot is required afterward."
} else {
    Write-Log "skipping NVIDIA driver step (pass -InstallDriver to include the NVIDIA App)"
    if (Get-Command nvidia-smi -ErrorAction SilentlyContinue) {
        $drv = (& nvidia-smi --query-gpu=driver_version --format=csv,noheader | Select-Object -First 1).Trim()
        Write-Log "current driver: $drv"
    } else {
        Write-Warn "nvidia-smi not found -- a driver may not be installed yet"
    }
}

# ---------------------------------------------------------------------------
# 3. CUDA Toolkit
# ---------------------------------------------------------------------------
Install-Winget 'Nvidia.CUDA' 'CUDA Toolkit'

# ---------------------------------------------------------------------------
# 4. Build tools
# ---------------------------------------------------------------------------
Install-Winget 'Kitware.CMake'      'CMake'
Install-Winget 'Ninja-build.Ninja'  'Ninja'
Install-Winget 'Git.Git'            'Git'

# ---------------------------------------------------------------------------
# 5. VS Code + extensions
# ---------------------------------------------------------------------------
if (-not $SkipVSCode) {
    Install-Winget 'Microsoft.VisualStudioCode' 'VS Code'

    # Refresh PATH so 'code' becomes visible in this session
    $env:PATH = [Environment]::GetEnvironmentVariable('PATH', 'Machine') + ';' +
                [Environment]::GetEnvironmentVariable('PATH', 'User')

    if (Get-Command code -ErrorAction SilentlyContinue) {
        Write-Log "installing VS Code extensions"
        foreach ($ext in @(
            'nvidia.nsight-vscode-edition',
            'ms-vscode.cpptools',
            'ms-vscode.cmake-tools')) {
            & code --install-extension $ext --force
        }
    } else {
        Write-Warn "'code' CLI not on PATH yet -- open a new shell, then run:"
        Write-Warn "  code --install-extension nvidia.nsight-vscode-edition"
        Write-Warn "  code --install-extension ms-vscode.cpptools"
        Write-Warn "  code --install-extension ms-vscode.cmake-tools"
    }
} else {
    Write-Log "skipping VS Code (-SkipVSCode)"
}

# ---------------------------------------------------------------------------
Write-Log "done."
Write-Host ""
Write-Host "Next steps:" -ForegroundColor Green
Write-Host "  1. Open Start menu -> 'x64 Native Tools Command Prompt for VS 2022'"
Write-Host "     (a plain PowerShell will NOT have cl.exe on PATH)"
Write-Host "  2. cd to the project directory, then:"
Write-Host "       cmake -S . -B build -G Ninja -DCMAKE_BUILD_TYPE=Release"
Write-Host "       cmake --build build"
Write-Host "       build\vector_add.exe"
Write-Host "  3. Verify the toolchain:"
Write-Host "       nvcc --version"
Write-Host "       nvidia-smi"
if ($InstallDriver) {
    Write-Host "  4. Reboot before running CUDA programs (driver was just updated)."
}

## 5. Project layout

cuda\
├── CMakeLists.txt           # CMake 3.24+, C++20, CUDA arch 61 (override for your GPU)
├── README.md                # quick-start
├── .clang-format            # Google base, 4-space, 100-col
├── .gitignore
├── docs\
│   └── SETUP_WINDOWS.md     # this document
├── scripts\
│   └── install.ps1          # automated environment installer (PowerShell)
├── include\
│   └── vector_add.cuh       # public kernel wrapper API
└── src\
    ├── main.cpp             # host driver, validates result
    └── vector_add.cu        # __global__ kernel + RAII-light wrapper

## 6. Build & run

Always use the “x64 Native Tools Command Prompt for VS 2022” (Start menu → search). A plain PowerShell or cmd will not have cl.exe on PATH and CMake will fail to find a host compiler.

cd C:\path\to\cuda
cmake -S . -B build -G Ninja -DCMAKE_BUILD_TYPE=Release
cmake --build build
build\vector_add.exe

### Override the GPU architecture for your laptop card

The CMakeLists.txt in this repo defaults to 61 (Pascal, GTX 1050 Ti) since that’s the desktop hardware it was scaffolded on. For a modern laptop GPU, override at configure time:

cmake -S . -B build -G Ninja -DCMAKE_CUDA_ARCHITECTURES=86

Common values:

| Architecture | Compute capability | Example cards |
| --- | --- | --- |
| Pascal | 61 | GTX 1050 Ti, 1080 |
| Turing | 75 | GTX 1660, RTX 2060 |
| Ampere | 80 / 86 | A100 (80) / RTX 30-series (86) |
| Ada | 89 | RTX 40-series |
| Hopper | 90 | H100 |
| Blackwell | 100 / 120 | RTX 50-series |

Find yours at https://developer.nvidia.com/cuda-gpus.

## 7. Verify the install

In an x64 Native Tools prompt:

nvidia-smi              :: driver visible, GPU listed
nvcc --version          :: should report 12.x
ninja --version
cmake --version         :: >= 3.24
cl                      :: should print Microsoft C/C++ Compiler banner
code --list-extensions  :: should include nsight, cpptools, cmake-tools

Then build and run; success looks like:

OK: 1048576 elements added on GPU

## 8. Driver rollback on Windows

If a new driver causes problems, Windows gives you three escalating recovery options. They map roughly to the Linux Timeshift / apt downgrade / nouveau tiers.

### 8.1 Roll back via Device Manager (gentlest)

Best when Windows still boots and the new driver is the obvious culprit.

- Start menu → Device Manager.

- Display adapters → right-click your NVIDIA GPU → Properties.

- Driver tab → Roll Back Driver.

- Pick a reason, click OK, reboot.

This restores the immediately previous driver Windows kept staged. If the button is greyed out, no previous driver is available — try §8.2.

### 8.2 System Restore (medium)

Windows automatically takes a restore point before driver installs (if System Protection is enabled — check Settings → System → About → Advanced system settings → System Protection).

- Start menu → “Create a restore point”.

- System Restore… → pick a point dated before the driver install → Next → Finish.

- Windows reboots into the restored state.

This reverts driver + registry changes without affecting your files.

### 8.3 DDU (Display Driver Uninstaller) — nuclear option

When the install is wedged badly enough that neither rollback method works:

- Download DDU from https://www.guru3d.com/files-categories/category/display-driver-uninstaller-ddu/.

- Boot Windows in Safe Mode (Settings → System → Recovery → Advanced startup → Restart now → Troubleshoot → Advanced options → Startup Settings → Restart → 4).

- Run DDU → Clean and shutdown.

- Boot normally; install a known-good driver (a specific version from https://www.nvidia.com/Download/index.aspx).

DDU removes every NVIDIA driver artifact — registry keys, files, scheduled tasks. Use only when you need to.

### 8.4 Pre-emptive caution

Before any driver upgrade:

- Settings → System → About → Advanced system settings → System Protection → confirm protection is On for C:.

- Note the current driver: nvidia-smi --query-gpu=driver_version --format=csv,noheader > driver-before.txt

- Manually create a restore point: Start → “Create a restore point” → Create.

## 9. WSL2 alternative

If you’d rather mirror the Linux setup exactly on this same Windows 11 machine:

wsl --install -d Ubuntu-24.04

Reboot, complete the Ubuntu setup, then inside WSL:

# Use the Linux install script
./scripts/install.sh

Important caveats:

- Install the Windows-side NVIDIA driver only. Do not install a Linux driver inside WSL — Microsoft’s WSL kernel ships its own driver shim that talks to the Windows driver. Installing a Linux driver inside WSL will break GPU access.

- The Windows driver must be a recent version that supports the WSL CUDA shim — any modern Game Ready / Studio driver does.

- nvidia-smi works inside WSL once you’ve installed CUDA in WSL — the GPU shows up as if it were native.

- Compute performance is within a few percent of bare Linux.

|  | Native Windows (this doc) | WSL2 |
| --- | --- | --- |
| Toolchain | MSVC | GCC (same as Linux) |
| Build identical to Linux? | No | Yes |
| GUI debugging via Nsight VSCE | Smoother | Works, slightly more setup |
| GPU driver lives where? | Windows | Windows (WSL borrows it) |
| When to pick | Tight Visual Studio integration | Mirror of Linux setup, lower friction if you already know Linux |

## 10. Common Windows-specific pitfalls

- MSVC ↔ CUDA version pairing. Each CUDA release supports a specific range of MSVC toolset versions. If you install the latest MSVC and an older CUDA toolkit, nvcc may bail with “unsupported Microsoft Visual Studio version.” The CUDA release notes list the supported MSVC range — match them, or pin an older toolset via the VS Installer’s Individual components tab.

- Wrong shell. A plain PowerShell or cmd.exe will not have cl.exe on PATH. Always build from the x64 Native Tools Command Prompt for VS 2022, or run vcvars64.bat first in your shell:

- "C:\Program Files (x86)\Microsoft Visual Studio\2022\BuildTools\VC\Auxiliary\Build\vcvars64.bat"

- Long paths. Some CUDA include paths exceed Windows’ classic 260-char limit. Either keep the project near the drive root (C:\src\cuda is good) or enable long paths via the registry:

- New-ItemProperty -Path "HKLM:\SYSTEM\CurrentControlSet\Control\FileSystem" `
  -Name "LongPathsEnabled" -Value 1 -PropertyType DWORD -Force

- Antivirus on the build directory. Real-time scanning of build\ can dramatically slow incremental builds. Add an exclusion for the project’s build folder if compilation feels sluggish (Windows Security → Virus & threat protection → Manage settings → Add or remove exclusions).

- Execution policy blocks install.ps1. PowerShell refuses to run unsigned local scripts by default. Use Set-ExecutionPolicy -Scope Process -ExecutionPolicy Bypass for the current shell only — no permanent system change.

- PATH not refreshed. After installing CUDA or VS Code via winget, the new PATH only takes effect in new shells. Close and re-open the terminal, don’t just retry in the same window.

- compute-sanitizer works on Windows too — same syntax: compute-sanitizer build\vector_add.exe. Always your first move when a kernel misbehaves.

## 11. Where to go next

- Tutorial: see docs/CUDA_TUTORIAL.md — 19 sections, all examples are platform-neutral.

- NVIDIA Programming Guide: https://docs.nvidia.com/cuda/cuda-c-programming-guide/

- Best Practices Guide: https://docs.nvidia.com/cuda/cuda-c-best-practices-guide/

- CUDA samples: https://github.com/NVIDIA/cuda-samples

- Nsight Compute training: https://developer.nvidia.com/nsight-compute